- General Machine Specifications

Machine type / category:

Submerged Arc Welding (SAW) System

Model / designation:

CaB 460C 3.0 × 6.0 m

Manufacturer:

ESAB

Year of manufacture:

2020

Control system (if applicable):

Integrated RCC remote control with PEK Digital Process Controllers (2 pcs); CAN-bus communication

Processing materials (types):

suitable for structural steel and heavy fabrication; wire diameter 3.0–6.0 mm

Typical workpiece dimensions (if applicable):

Vertical reach: 950–3,950 mm; Horizontal reach: 715–6,715 mm from column centre

Max part weight (if applicable):

5 tonns

Power / capacity / tonnage (if applicable):

2 × ESAB LAF 1001 DC power sources; 1,000 A at 60% duty cycle / 800 A at 100% duty cycle each

Other (please specify):

Track width: 1,730 mm inside-to-inside; Total height: 5,275 mm (from rail surface); Supply voltage: 3×400 VAC + N + PE / 50 Hz

- Included Options and Equipment

[x ]  Dust collector (model, if known): Flux system: FFRS Super with heaters and TEDAK filter; TPC 75 flux tank (75 kg capacity); 2× OPC Super flux recovery units; pneumatic control from RCC

[ ] Loading equipment (e.g., conveyor):

[ ] Unloading equipment (e.g., conveyor):

[x ]  Sensors or automation options: 2× GMH joint tracking system (motorised servo slide, 180×180 mm stroke per head); flux low-level sensor with warning light

[ x] Measurement equipment: Measurement equipment: PEK Digital Process Controller (2 pcs) – logs min/max/avg current, voltage, travel speed and energy per unit length for up to 100 welds

[x ] Additional functions (e.g., short part handling, centering): Tandem welding (20-Tandem 6×6); shift function between boom and horizontal slide; transport speed 5 m/min; pneumatic brake; manual column rotation; dual camera system with PiP (15" LCD monitor)

[x ] Software (CAD/CAM, control systems): PEK controller – 255 weld data sets; production statistics; 3 user access levels; USB data port; 16 languages

[x ]   Manuals and documentation: Documentation and factory testing included per scope of supply

Other (please specify):

- Customised double A6S SAW HD 156:1 welding head on 800 mm beam

- Horizontal slide with 358 mm stroke

- 2× welding rectifiers LAF 1001 with all welding cables (27 m and 15 m, 95 mm² area)

- Rear-mounted wire reel holder for 2×30 kg spools with wire liners

- 2× pilot laser lamps (25 m cord each)

- Single platform (no operator cab) at column base with power sources mounted

- Machine Condition and History

Condition (tick one):

[x]  Excellent

[ ] Used, in good condition

[ ]  Requires repair

Is the machine fully operational?

Yes

Operating hours (if available):

500h

Serial number:

CAB460C-A102457, Welding units 536-039-5964 / 536-038-5792

Other (please specify):

CE certification:

[x]  The machine is equipped with CE certification

[ ]  The machine does not have CE certification

Status of the device:

[x]  The machine is currently in operation

[ ]  The machine is currently idle

- Commercial Details

Machine price EUR :

80000eur without VAT (disassembly and transportation not included)

Delivery terms:

FCA, Ventspils

Warranty terms:

No warranty

Machine location (city, country):

Ventspils, Latvia

- Attachments

  - Machine photos (general view, control panel, key components)

  - Technical documentation (if available)